LABORATORY REPORT

IN

CONTROL ENGINEERING

Submitted by:

Dan Mark C. Pastoral

Submitted to:

Engr. Miguel Albert Calizar

LABORATORY ACTIVITY NO. 05
Pneumatic Systems and Applications

OBJECTIVE:

- To be familiar with the pneumatic systems and its applications.

ACTIVITY:

- What is a pneumatic system?

- Site some advantages, limitations, and explain some components in a pneumatic system.

- Give some applications of a pneumatic systems.

ANSWER:

1. What is a Pneumatic System?

A pneumatic system is a type of power transmission system that uses compressed air or gas to generate, transmit, and control mechanical force and motion. The word “pneumatic” is derived from the Greek word “pneuma,” meaning breath or air. In a pneumatic system, a compressor draws in atmospheric air, compresses it to a higher pressure, and stores it in a reservoir or receiver tank. This pressurized air is then distributed through a network of pipes and hoses to actuators such as pneumatic cylinders and motors, which convert the compressed air energy into mechanical work. Pneumatic systems are widely used in industrial automation, manufacturing, and transportation because compressed air is readily available, clean, safe, and easy to control.

[ Illustration 1: Basic Pneumatic System Diagram ]

2. Advantages of Pneumatic Systems

a. Safety – Compressed air is non-flammable and non-toxic, making pneumatic systems inherently safe to use in hazardous environments such as chemical plants, paint booths, and food processing facilities where sparks or oil leaks from hydraulic systems could be dangerous.

b. Cleanliness – Since the working medium is air, there is no risk of fluid contamination. Exhaust air is simply released into the atmosphere, which is ideal for industries with strict hygiene standards such as food and pharmaceutical manufacturing.

c. High Speed Operation – Pneumatic actuators can operate at very high speeds compared to hydraulic and electric systems, enabling rapid repetitive movements required in assembly lines and packaging machines.

d. Simplicity and Low Maintenance – Pneumatic components are relatively simple in design, easy to install, and require less maintenance compared to hydraulic systems. There is no need for return lines since exhaust air is vented to the atmosphere.

e. Energy Storage – Compressed air can be stored in receiver tanks and used on demand, providing a readily available power source even during peak demand periods without overloading the compressor.

3. Limitations of Pneumatic Systems

a. Limited Force Output – Pneumatic systems operate at relatively low pressures (typically 6 to 10 bar), which limits the force they can generate. They are not suitable for heavy-load applications that require very high forces, unlike hydraulic systems that can operate at pressures exceeding 300 bar.

b. Compressibility of Air – Air is compressible, which makes precise speed and position control more difficult compared to hydraulic systems. The compressibility causes jerky or erratic movements in some applications requiring fine motion control.

c. Noise – The exhaust of compressed air produces significant noise, which may require the use of silencers or mufflers in work environments to comply with occupational health and safety standards.

d. Energy Inefficiency – Compressing air consumes a large amount of electrical energy, and a significant portion is lost as heat during compression. The overall energy efficiency of pneumatic systems is relatively low compared to electric drive systems.

4. Components of a Pneumatic System

a. Air Compressor

The air compressor is the heart of a pneumatic system. It draws in atmospheric air and compresses it to the required operating pressure. Common types include reciprocating (piston), rotary screw, and centrifugal compressors. The compressed air is then stored in a receiver tank for distribution.

[ Illustration 2: Air Compressor ]

b. Receiver Tank (Air Reservoir)

The receiver tank stores the compressed air produced by the compressor. It acts as a buffer to handle sudden demands, reduce pressure fluctuations, and allow condensed moisture to settle before the air enters the distribution system. It is typically equipped with a pressure gauge, safety relief valve, and drain valve.

[ Illustration 3: Receiver Tank / Air Reservoir ]

c. FRL Unit (Filter, Regulator, Lubricator)

The FRL unit is an air preparation assembly installed between the compressor and the pneumatic circuit. The Filter removes dust, water droplets, and contaminants from the compressed air. The Regulator reduces and stabilizes the supply pressure to the required working pressure. The Lubricator adds a fine mist of oil to the air stream to lubricate downstream components such as valves and cylinders, extending their service life.

[ Illustration 4: FRL Unit (Filter-Regulator-Lubricator) ]

d. Directional Control Valve (DCV)

The directional control valve controls the direction of airflow to and from the pneumatic actuator. It determines whether the piston in a cylinder extends or retracts by switching the flow path of compressed air. Common types include the 3/2-way valve (3 ports, 2 positions) and the 5/2-way valve (5 ports, 2 positions). DCVs can be actuated manually, mechanically, pneumatically, or electrically via solenoids.

[ Illustration 5: 5/2-Way Directional Control Valve ]

e. Pneumatic Cylinder (Actuator)

The pneumatic cylinder is the primary output device (actuator) in a pneumatic system. It converts the energy of compressed air into linear mechanical motion. A double-acting cylinder uses compressed air on both sides of the piston to produce force in both the extension and retraction strokes, while a single-acting cylinder uses air pressure only for one stroke with a spring return for the other. Pneumatic cylinders are commonly used for clamping, pressing, lifting, and pushing operations in automated machinery.

[ Illustration 6: Double-Acting Pneumatic Cylinder ]

5. Applications of Pneumatic Systems

a. Manufacturing and Assembly Lines – Pneumatic cylinders and grippers are extensively used in automated assembly lines for tasks such as part transfer, pressing, clamping, and ejection. The speed and repeatability of pneumatic actuators make them ideal for high-volume production environments.

b. Braking Systems in Vehicles – Pneumatic braking systems are used in heavy commercial vehicles such as trucks, buses, and trains. Air brakes provide reliable and consistent stopping power even under heavy loads, and fail-safe spring brakes engage automatically when air pressure is lost.

c. Pneumatic Tools – Air-powered tools such as pneumatic drills, impact wrenches, nail guns, spray guns, and sandblasters are widely used in construction, automotive repair, and woodworking industries. These tools are lighter, more powerful, and more durable than their electric equivalents for sustained heavy-duty use.

d. Food and Beverage Industry – Pneumatic systems are used for filling, sealing, labeling, and packaging operations in food and beverage plants. Because compressed air is clean and oil-free (when properly filtered), it is safe for use in environments where contact with food products is possible.

e. Medical Equipment – Pneumatic systems are used in ventilators, dental drills, surgical tools, and patient transfer systems in the healthcare industry. The clean and controllable nature of compressed air makes it suitable for sensitive medical applications where precision and hygiene are critical.

REMARKS:

REFERENCES:

Bosch Rexroth AG. (2019). Pneumatics: Basic Level. Lohr am Main: Bosch Rexroth AG.

Parr, A. (2011). Hydraulics and Pneumatics: A Technician’s and Engineer’s Guide (3rd ed.). Oxford: Butterworth-Heinemann.

SMC Corporation. (2020). Pneumatic Components and Systems Catalog. Tokyo: SMC Corporation.